# FPGA DEVELOPMENT GUIDE

Gate 0 Implementation Reference

AMD Kria KV260 Platform | March 2026

# 1. Overview

This guide provides the complete implementation reference for the Gate 0 FPGA demonstration on the AMD Kria KV260 platform. The target is to achieve 25+ tokens/s throughput at less than 5W power consumption using the BitNet b1.58-2B-4T model. The implementation is based on the validated TeLLMe v2 reference design (arXiv:2510.15926) which has demonstrated these specifications on the same hardware platform.

# 2. Hardware Setup

## 2.1 Required Equipment

| Item | Quantity | Est. Cost |
| --- | --- | --- |
| AMD Kria KV260 Starter Kit | 3 | $300 each = $900 |
| USB-C Power Supply (5V/4A) | 3 | $20 each = $60 |
| microSD Cards (64GB) | 3 | $15 each = $45 |
| USB-UART Cable | 3 | $15 each = $45 |
| Power Meter (USB) | 1 | $50 |
| Total Hardware Budget | - | $1,100 |

## 2.2 Platform Specifications

The AMD Kria KV260 features a Zynq UltraScale+ XCK26 MPSoC with dual-core ARM Cortex-A53 processor, dual-core ARM Cortex-R5 real-time processor, and programmable logic with 246K system logic cells, 368 DSP slices, and 14.4Mb block RAM. The onboard 4GB LPDDR4 memory provides sufficient capacity for KV cache and activation storage during inference operations.

# 3. Development Environment

## 3.1 Software Requirements

- Vitis Unified IDE 2023.2 or later (free WebPACK license)

- Vivado Design Suite 2023.2 or later

- PYNQ framework v3.0.1 for KV260

- Python 3.10+ with NumPy, PyTorch, transformers

- Git for version control

- bitnet.cpp repository (Microsoft)

## 3.2 Environment Setup Steps

- Download and install Vitis/Vivado from AMD Xilinx website (requires free account)

- Flash KV260 with PYNQ 3.0.1 image from official repository

- Clone bitnet.cpp: git clone https://github.com/microsoft/BitNet.git

- Download BitNet b1.58-2B-4T weights from HuggingFace

- Build bitnet.cpp for ARM: cmake -DLLAMA_CUBLAS=OFF && make

# 4. Implementation Architecture

## 4.1 TeLLMe Reference Architecture

The implementation follows the TeLLMe v2 architecture which achieves 25 tok/s at 4.8W on the KV260 platform. The core innovation is the Table-Lookup Matmul (TLMM) engine that precomputes all possible ternary multiplication results and stores them in FPGA LUTs as lookup tables. This approach eliminates the need for hardware multipliers, significantly reducing resource utilization and power consumption.

## 4.2 Resource Requirements

| Resource | Utilization | Available |
| --- | --- | --- |
| LUTs | 98K (40%) | 246K |
| DSP Slices | 610 (166%) | 368 |
| Block RAM | 60 URAM | 14.4Mb |
| Clock | 250 MHz | Target |

Note: DSP slice utilization exceeds available resources on the KV260 for the full 2B model. The TeLLMe implementation uses LUT-based multiplication for overflow, accepting slight performance reduction.

# 5. Performance Targets and Validation

## 5.1 Gate 0 Success Criteria

| Metric | Target | Validation Method |
| --- | --- | --- |
| Decoding Throughput | ≥25 tok/s | Benchmark script |
| Power Consumption | ≤5W | USB power meter |
| Model Accuracy | ≤5% degradation | MMLU benchmark |
| Latency (first token) | <100ms | Timing measurement |

# 6. Development Timeline

## 6.1 Week-by-Week Plan

Week 5: Environment Setup and Model Preparation

- Unbox and configure KV260 boards

- Install Vitis/Vivado development environment

- Download and verify BitNet b1.58-2B-4T weights

- Build bitnet.cpp and validate CPU inference

Week 6: RTL Development and Initial Synthesis

- Implement TLMM engine in Verilog/SystemVerilog

- Create systolic array structure

- Initial synthesis targeting KV260

- Resolve timing violations

Week 7: Integration and Debugging

- Integrate RTL with ARM PS software stack

- Implement KV cache management

- Debug timing and functional issues

- First inference run

Week 8: Optimization and Validation

- Performance optimization for 25 tok/s target

- Power measurement and optimization

- Accuracy benchmarking (MMLU)

- Documentation and demo preparation

# 7. Key Resources and Contacts

- TeLLMe Authors: Contact via arXiv:2510.15926 for implementation details

- Tong Yang (PKU): tongyang@pku.edu.cn - iFairy hardware collaboration

- KAIST HPIC Lab: https://hpic-lab.github.io - 2T1C DRAM research

- BitNet GitHub: https://github.com/microsoft/BitNet

- KV260 Documentation: https://docs.xilinx.com/r/en-US/ug1089-kv260-starter-kit